Michigan Department of Health and Human Services

Bureau of Laboratories

# DCH-0597 Instructions for Submission of Isolates for Bacterial, Fungal, and Mycobacterial Identification

Rev 06/01/2024

## Collection Kit

Unit 42B (IATA Category B).

Unit 42A (IATA Category A): Shipped only by certified Category A trained personnel. Please contact our DASH unit at 517-335-8059 if assistance is needed.

## Acceptable Isolates

- Aerobic bacterial isolates submitted on enriched medium suitable for growth.

  - NOTE: Potential Select Agents (B. anthracis, Brucella sp., Burkholderia mallei/pseudomallei, Clostridium botulinum, Francisella tularensis, and Yersinia pestis) and known isolates of Shiga-toxin producing E. coli must be shipped Category A.

- Fungal and yeast isolates submitted on agar slants.

  - NOTE: Known isolates of Coccidioides immitis must be shipped Category A.

- Mycobacterial isolates in broth or slanted media.

  - NOTE: Known isolates of Mycobacterium tuberculosis must be shipped Category A.

- Broth cultures for Bacterial and Fungal identification are accepted only by prior notification.

- Petri plates for Bacterial and Fungal identification are accepted only by prior notification.

## Labeling

- Label the specimen with the complete patient’s name and at least 1 unique identifier (specimen number, medical record number, etc.).

- Enter the required information on the test requisition form (DCH-0583). The patient’s name and unique identifier on the form must be entered exactly as the information on the tube.

- If there are multiple specimens from the same patient, please label each specimen tube with the correct specimen number. A separate test requisition is needed for each specimen.

## Packaging and Shipping - Category B

- Make sure the cap on the tube is seated tightly and evenly to prevent leaking. Para-film® or tape the cap to secure.

- Wrap the isolate with paper toweling and insert the isolate with the absorbent pad into the small plastic bag provided. Seal the bag.

- Place the small bag in the larger Category B secondary container (plastic bag with the biohazard symbol that comes with the Category B shipping materials supplied by MDHHS Bio-Bag). Seal the bag by removing the white strip and folding over the adhesive flap of the bag.

  - NOTE: If multiple samples will be shipped in the same canister, all samples must be individually wrapped and separated to prevent contact between them.

- Place the completed MDHHS test requisition in the outside pocket of the Bio-Bag.

- Insert the Bio-Bag diagonally into the provided Category B box so it is suspended in the middle. If necessary, add extra cushioning such as paper toweling or bubble wrap).

- Close the box and secure with sealing tape on both sides of the flap.

- Attach completed address label and the UN3373 label if not stamped on the box.

- Send to the Michigan Dept. of Health and Human Services as soon as possible (e.g., courier, 1st class mail (USPS), United Parcel Service (UPS), etc.)

- Instructions are also available by visiting www.Michigan.gov/mdhhslab.

- The specimen will not be tested if:

- The specimen is received leaking or broken.

- The specimen is not properly labeled, or the test requisition not completed.

- The specimen label does not match the test requisition.

## Packaging and Shipping – Category A

- Make sure the cap on the tube is seated tightly and evenly to prevent leaking. Para-film® or tape the cap to secure.

- Wrap an absorbent pad such as paper toweling around the isolate slant and insert the specimen with absorbent pad into the small bubble wrap pockets provided. Seal the flap.

- Wrap the small bag with extra cushioning material (padding) to avoid specimen jostling and insert in the Category A secondary container (white with red lid) MDHHS. Rotate the lid until locked in place.

  - NOTE: If multiple samples will be shipped in the same canister, all samples must be individually wrapped and separated to prevent contact between them. (50ml maximum per canister).

- Place the completed MDHHS test requisition around the outside of the canister and insert in the foam holder located inside the Category A box.

- Close the box and secure with sealing tape around the sides of the flap.

- Complete the Dangerous Goods Form (DGF).

  - NOTE: Make 4 copies of the DGF (3 are kept with the specimen for the carriers use and the 4th copy is retained at your facility for 2 years based on DOT regulations).

- Fold the dangerous good forms in half and insert in the clear adhesive pocket. Affix to the side of the box.

- Attach completed address label, the Class 6 label and UN 2814 label if not stamped on the box or already attached.

- Record the volume of the sample on the outside of the box .

- Record the name and telephone number of the person responsible on the outside of the package.

- Send to the Michigan Department of Health and Human Services as soon as possible (e.g. courier, Fed-Ex, UPS)

  - NOTE: CATEGORY A CANNOT BE SENT BY USPS.

- Instructions are also available by visiting Michigan.gov/mdhhslab.

- The specimen will not be tested if:

1.	The specimen is received leaking or broken.

2.	The specimen is not properly labeled, or the test requisition not completed.

3.	The specimen label does not match the test requisition.

## Ensuring packaging and shipping follows all applicable regulations is the responsibility of the shipper

Kit 42B – Bacterial and Fungal Cultures (Submission of Category B Infectious Substances – UN3373

Kit 42A: Bacterial and Fungal Cultures (Submission of Category A Infectious Substances - UN 2814)