MICHIGAN DEPARTMENT OF ENVIRONMENT, GREAT LAKES, AND ENERGY

Materials Management Division

# Form EQP5111 Attachment Template A5 – Inspection Requirements

This document is an attachment to the Michigan Department of Environment, Great Lakes, and Energy’s Instructions for Completing Form EQP5111, Operating License Application Form for Hazardous Waste Treatment, Storage, and Disposal Facilities.  See Form EQP5111 for details on how to use this attachment.

The administrative rules promulgated pursuant to Part 111, Hazardous Waste Management, of Michigan’s Natural Resources and Environmental Protection Act, 1994 PA 451, as amended (Act 451), being R 299.9504, R 299.9508, R 299.9605 and Title 40 of the Code of Federal Regulations (CFR) §§264.15 and 270.14(b)(5), establish requirements for inspections at hazardous waste management facilities.  All references to 40 CFR citations specified herein are adopted by reference in R 299.11003

This license application template addresses requirements for inspections at the following hazardous waste management facility: [Facility Name] in [City], Michigan.

(Check as appropriate)

Applicant for Operating License for Existing Facility

Applicant for Operating License for New, Altered, Enlarged, or Expanded Facility

This template is organized as follows:

INTRODUCTION

A5. A     WRITTEN SCHEDULE

A5. A.1     Types of Problems

A5. A.2     Frequency of Inspection

A5. B     REMEDY SCHEDULE

A5. C	   INSPECTION LOG OR SUMMARY

Table A5. C.1 Container Storage Area Inspection Log Example

## INTRODUCTION

- The owner or operator of a hazardous waste management facility must inspect the facility for malfunctions and deterioration, operator errors, and discharges that may be causing, or may lead to: (1) release of hazardous waste constituents into the environment or (2) a threat to human health.  The owner or operator must conduct this inspection often enough to identify problems in time to correct them before they harm human health or the environment [R 299.9605 and 40 CFR §264.15(a)].

## A5. A     WRITTEN SCHEDULE

[R 299.9605 and 40 CFR §264.15(b)(1)]

- To meet the inspection requirements, develop and follow a written schedule for inspecting monitoring equipment, safety and emergency equipment, security devices, and operating and structural equipment (e.g., as dikes and sump pumps) that are important to preventing, detecting, or responding to environmental or human health hazards.  This written inspection schedule must be kept at the facility.

### A5. A.1     Types of Problems

[R 299.9605 and 40 CFR §264.15(b)(3)]

F	Identify the types of problems (e.g., malfunctions or deterioration) that are to be looked for during the inspection (e.g., inoperative sump pump, leaking fitting, eroding dike, etc.).

### A5. A.2     Frequency of Inspection

[R 299.9605 and 40 CFR §§264.15(b)(4), 264.174, 264.193, 264.195, 264.226, 264.254, 264.278, 264.303, 264.347, 264.602, 264.1033, 264.1052, 264.1053, 264.1058, and 264.1083 through 264.1089, where applicable]

F	The minimum frequency of inspection should be based on the requirements for each unit on the written schedule.  However, the actual frequency of inspection may be more frequent than the minimum and should be based on the rate of deterioration of the equipment and the probability of an environmental or human health incident if the deterioration, malfunction, or any operator error goes undetected between inspections.  Areas subject to spills (e.g., loading and unloading areas) must be inspected daily when in use.  At a minimum, the inspection schedule must include the items and frequencies called for in the applicable regulations specified above.  Additionally, other attachment templates may refer to inspection requirements.  Those inspection requirements should be specified in this template and referenced in other attachment templates.

## A5. B     REMEDY SCHEDULE

[R 299.9605 and 40 CFR §264.15(c)]

F	Include a written schedule to remedy any deterioration or malfunction of equipment or structures that the inspection reveals that ensures that the problem does not lead to an environmental or human health hazard.  Where a hazard is imminent, or has already occurred, remedial action must be taken immediately.

## A5. C     INSPECTION LOG OR SUMMARY

[R 299.9605 and 40 CFR §264.15(d)]

F	The owner or operator must record inspections in an inspection log or summary.  Copies of these records must be kept for at least three years from the date of inspection.  At a minimum, these records must include the date and time of the inspection, the name of the inspector, a notation of the observations made, and the date and nature of any repairs or other remedial actions taken.  An example log is provided as Table A5. C.1.

### Table A.5. C.1     Container Storage Area Inspection Log Example

Date/Time: [insert date and time]	Name of Inspector: [insert full name]

| Inspection Item | Observations | Date of Repairs | Nature of Repairs | Date of Remedial Actions | Nature of Remedial Actions | Inspection Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| Operator Errors | None |  |  |  |  | Weekly |
| Discharges that may be causing or may lead to: (1) release of hazardous waste constituents to the environment or (2) a threat to human health and the environment. | None |  |  |  |  | Weekly |
| Leaking Containers | Leaking 55-gallon drum. About 1 gallon pooled in secondary containment from F005. | Same Day | Overpacked drum, sorbent used on spill | NA | NA | Weekly |
| Container Deterioration | 1 Container | Same Day | Overpacked drum, sorbent used on spill | NA | NA | Weekly |
| Containment System Deterioration | None |  |  |  |  | Weekly |

_______________________________________________________________________________

People with disabilities may request this material in an alternate format by emailing EGLE-Accessibility@Michigan.gov or calling 800-662-9278.

EGLE does not discriminate on the basis of race, sex, religion, age, national origin, color, marital status, disability, political beliefs, height, weight, genetic information, or sexual orientation in the administration of any of its programs or activities, and prohibits intimidation and retaliation, as required by applicable laws and regulations.

This form and its contents are subject to the Freedom of Information Act and may be released to the public.