Construction I & II Math Alignment

Modern Carpentry:

# Unit 1

- Pg. 20—“lumber is cut so that the annular rings form an angle of less than 45 degrees w/the surface of the board”

- Pg. 20—formula for moisture content of wood; uses % and temp. measurements in the calculation

- Pg. 21—Equilibrium moisture content; state where the moisture in the wood is balanced with that in the air; comparing percentages and making decisions based on that comparison

- Moisture Meters—require knowing the range and accuracy of the measurement; using percentages again (Discuss ONLY)

- Grading standards for lumber require knowing the required characteristics, which items fall within these specifications and which do not; some of these standards revolve around measurements or percentages

- Calculating board footage requires multiplication with fractions and cross cancellation

- Standard Lumber Sizes Table on pg. 26 and Figure 1-21—both require extensive understanding of measurement and units

- Figure 1-47; Nail types and specifications for securing connectors and hangers; uses skills of measurement, use of fractions, decimals, %, and rates

- Given the characteristics of a piece of wood, identify which type it might be (like identifying which geometric figure fits the listed properties)

# Unit 2

- “Angle of Repose” in an excavation site

# Unit 3

- Setting a T-Bevel at a specified angle

- Combination square used for making “parallel lines” as well as “90 degree and 45 degree angles”

- Level and Plumb Bob used for laying out vertical and horizontal lines

- Using a compass to draw arcs and circles (figure 3-4 on pg. 57)

- Angles on the blades of a crosscut saw and a rip saw (Figure 3-11)

- Dimensions of each tool and measurements of types of cuts that some make continue to require an extensive knowledge of fractions and English Measurement System

# Unit 4

- Several rates used such as rpm’s or teeth per inch on a saw blade

- Continuous use of various units of measurement, including fractional amounts

- Cutting along “oblong or circular” pieces (pg. 87)

- Both orbital and oscillating sander concepts

- Angles the carpenter is able to cut using a miter saw

# Unit 5

- Perpendicular lines used in measurements, pg. 101

- “6-8-10” Method for locating lines (special triangles)

- Plot plan on pg. 102 requires use of various measurements, parallel and perpendicular lines, and ratio and proportion; scale drawings

- Using “lines of sight” with leveling instruments

- Discussion of vertical planes and vertical angles

- Using various measuring tapes that use fractional and decimal amounts as well as different units

- Figure 5-6; Level-transit used to measure angles in either horizontal or vertical planes

- Process of “sighting” requires knowledge of angles, similar triangles, horizontal and vertical lines, slope (grade levels and elevations), and measuring skills

- Laying out corners with the transit and the “Horizontal Graduated Circle”; uses concept of degrees, quadrants, arcs and how many degrees the arcs are, etc.

- Reading a vernier scale

- Staking out a building is similar to the construction of various shapes in traditional Geometry class using compass and straightedge

- Pg. 110—using a level transit to measure vertical angles, establish a vertical line, also understanding concept of a vertical plane

# Unit 6

- Scale drawings in floor plans require constant use of rate and ratio

- Representing 3-D figures

- Use of isometric sketch paper (concept of an isometry)

- Repeated use of slope and Pythagorean Theorem in sketches for a home as well as actual construction

- Addition and subtraction of units (8’ 4” – 6’ 10” requires “borrowing” from the 8’)

- Listing specifications in Design Live Loads such as Floors---40 PSF

- Continuous application of area concepts, including irregularly shaped figures

# Unit 7

- Diagonals of a square or rectangle will always be equal—pg. 144; using this concept to check building lines

- Continued requirements as laid out by building codes, specifications, etc. require the student to understand degree of accuracy and make conclusions about the consequences of not falling within range

- Students must continuously use concept of perimeter, including irregular shaped figures

- Sectional views—pg. 165, Figure 7-53

- Table for protecting concrete; gives acceptable ranges and how to handle each temp. range to protect the concrete

- Formula for cubic yards in estimating materials

- Many procedures involve the practice of estimation

# Unit 8

- Sizing Girders—process that involved midpoints, total load/ft.2, staying within a particular range of values, multiplication of various measurements taken, and making conclusions based on your calculations

# Unit 9

- Pg. 226; Figure 9-51—Using the framing square to lay out a trim cut; finding slope

- Pg. 229—Formula for wall and total plate material requires use of many operations, including adding a certain percentage to your initial total

- Formulas for total number of studs, number of ceiling joists, net area of wall sheathing, and number of fiberboard sheets needed to complete a job

# Unit 10

- Students must be familiar with and have a fairly good estimate of various common angle measures (30,45,60,90,180, etc…)

- Pythagorean Theorem as well as recognize the base, altitude, and hypotenuse of a right triangle

- Understanding squares and square roots

- Diagonals of a 12” square are 16.97” (45-45-90 triangle formed inside square); diagonal is 12 X square root of 2

# Unit 11

- “Sequence of operations” that must be followed in roof construction; “order of operations”

- More estimation necessary when selecting roofing materials to do the job

- “Square”—the amount of roofing material needed to provide 100 ft.2 of finished roof surface; unit of measurement for estimating and purchasing roofing materials

- More estimating of roofing materials; involves slope and a percentage of the area

# Unit 12

- R-Values of a pane of glass; understanding what the decimal is what it means; understanding how to compare it to other R-values

- Sectional Drawings of windows (pg. 326-327)

- Recommended clearances for sealed insulating glass; use of tolerances; +/- amounts

# Unit 13

- Process for estimating siding involves using a constant (direct variation); also involves use of percentages to be added when covering triangular areas or areas with many corners

# Unit 14

- Calculating “U”, total heat transmission

- Understanding how changing an R-value affects the U-value

- R-Values can be converted to U-values by calculating the reciprocal

- Relationship between heat transmission and insulation thickness; Window thickness and U-values

- Tables on insulation coverage information on pg. 402 require estimation and calculation when purchasing insulation

- Multi-step formula/process for estimating the amount of insulation for exterior walls (Add up total perimeter of structure, multiply by ceiling height, deduct from the total the area of doors and windows)

- Decibel scale and terms related to sound in order to insulate homes acoustically

- Figure 14-59; interpreting graphs that represent the transmission loss values in decibels at various frequencies

- More estimation involved in materials needed for ceiling finish; calculating area, using a factor to multiply by in some cases, and adding a percentage for waste in some cases

# Unit 16

- Chalk lines for laying out flooring (figure 16-17) demonstrates 3-4-5 triangle and dividing a 45-45-90 triangle into two smaller 45-45-90 triangles

- Necessary to find the “midpoint” of the end walls of a room to lay tile; this skill required in several other areas of the construction process

# Unit 17

- Stairwell construction requires understanding of terms such as “geometrical” or “circular” stairs for winding staircases; also rise, run, vertical, and horizontal

- Preferred angle with floor is 30-35 degrees

- Stair calculations to determine the riser height and total run involve measurement, estimation, dividing and rounding

# Unit 22

- Transverse and longitudinal beams

- Recognize arch types such as radial and parabolic (figure 22-31)

# Unit 30

- Multiple statistics on the job outlook for construction trades

Math Used Repeatedly Throughout Text:

- Dimensions of a house

- Area of rectangles w/missing pieces (area of irregular shapes; finding missing lengths)

- Repeatedly using formulas for area of basic geometric shapes

- Finding the total cost of cement if dimensions are 100’ X 10’ and cost of cement is $3/ft.2

- Profit of the guy doing the job is not $3,000 (from example above); What are his other costs? (Materials, other labor, equipment, food, etc.)

- If concrete is $80/yd and one yd covers 80 ft.2, then what will it cost to cover 1000 ft.2?

- If you pay $1,000 for the concrete and the customer pays you $3/ft.2 for the job, what is your profit?  Again, what is the actual profit considering additional costs?

- Estimation

- Interpretation of data as it is represented in MANY forms (charts, tables, graphs)

- Drawing various “views” of 3-D figures—lateral, front, rear, etc.

- Vertical Angles, Alternate Interior, Alternate Exterior, Linear Pair, etc. all demonstrated in construction of a house

- Repeated use of area and volume as well as rates and proportion concepts with area and volume (pg. 385, for example, “parts by volume”)

- Repeated use of the concept of congruency; figures, angles, line segments, etc.

- Repeated use of the concept of similarity

- Testing a hypothesis and changing variables to produce desired results; “troubleshooting”

- Using rules, tapes, and squares

- Adding, subtracting, multiplying, and dividing fractions

- Finding common denominators

- Adding, subtracting, multiplying, and dividing decimals

- Converting between decimals and fractions

- Reducing fractions to lowest terms

- Converting between metric and English measurements

- Using squares and square roots

- Changing ratios to percents and vice versa

- Changing percents to fractions

- Converting necessary when using ratio, proportion, and percent

- Business math concepts such as finding a selling price, calculating profit margin, etc.

- Setting up equations based on the given data (ex: a 12 ft. board is sawed in two so that one part is 4 times the other, what is the length of each part?)

- Order of operations

- Bisecting an angle

- Knowing characteristics of isosceles and right triangles

- Finding an altitude in a triangle

- Both rotational and line symmetry of figures

- Understanding the term diameter and use of it in SEVERAL settings