Overhead Crane Permit Qualification Test 
Answer Sheet

- (b) False.  There are several types of overhead cranes.  Some are top running, some are under-hung, and some are semi gantry and gantry cranes.

- (b) Under-hung.

- (a) Perform a daily crane inspection.  R.1871(2)

- (d) All of the above.  See Table 2.

- (a) True.  The upper limit switch should not be used as an operating control, unless the hoist is equipped with a backup limit switch.  R.1861(2)

- (c) 2.  R.1861(3)

- (d) Tag the crane out of service and report the defect to your supervisor.  R.1857

- (a) True.  R.1865(1)

- (a) 10 degree.  R.1825(b)

- (c) Permanently identified by function and direction.  R.1843(1)

- (a) Both Sides.  R.1822(1)

- (b) False.  Never remove a tag or operate a crane that has been tagged out of service.

- (b) Warn other employees in the area of crane movement.  Cranes can be silent killers if someone is distracted or not paying attention to their surroundings.  Warning lights and sirens have helped reduced injury and fatalities in the workplace.  R.1845(1)

- (b) False.  This will actually create more swing, and can cause damage to the contactors.  Just slowly decelerate the crane by releasing pressure on the travel button ahead of your intended destination.  R.1865(6)(g)

- (d) Either (a) or (b).  All shipments are weighed before they are transported.  Check the shipping documents or the manufacturer’s tag, before hoisting the load.  Weighing the load is not a good choice in this case because it would require you to ether lift the load or the driver would have to back weigh the load, and one is time consuming and the other is unsafe.

- (b) False.  R.1825(2)

- (d) 125 percent.  R.1873(4) or R.1874

- (c) Always.

- (d) (a) and (b).  R.1851(1)

- (a) True.  R.1847(3)

- (c) Only if established by a qualified person.  R.1865(2)

- (d) Both (a) and (c).

- (c) The Wire Rope  R.1824(1)(b)

- (d) All of the above

- (b) False.  The load should only be carried high enough to clear obstacles  R.1865(6)(c)

- (b) False.

- (d) Either (a) or (c).  Most safety latches are designed to spring up when the hook exceeds 15 degrees of stretch and/or the safety latch is not the correct size.

- (b) Brake is adjusted too loose.

- (b) False.  R.1808(7)

- (a) True.  R.1859(2)

Contact:  	Consultation Education & Training (CET) Division

Michigan Occupational Safety & Health Administration (MIOSHA)

(517) 284-7720 or www.michigan.gov/miosha

LARA is an equal opportunity employer/program.

Auxiliary aids, services and other reasonable accommodations are available upon request to individuals with disabilities.