DEPARTMENT OF LICENSING AND REGULATORY AFFAIRS

DIRECTOR'S OFFICE

CARNIVAL AND AMUSEMENT SAFETY

GENERAL RULES

Filed with the secretary of state on

These rules take effect immediately upon filing with the secretary of state unless adopted under section 33, 44, or 45a(9) of the administrative procedures act of 1969, 1969 PA 306, MCL 24.233, 24.244, or 24.245a.  Rules adopted under these sections become effective 7 days after filing with the secretary of state.

(By authority conferred on director of the department of licensing and regulatory affairs by section 6 of 1966 PA  225, MCL 408.656, and Executive Reorganization Orders Nos. 1991-9, 1996-2, 2003-1, 2008-4, and 2011-4, MCL 338.3501, 445.2001, 445.2011, 445.2025, and 445.2030.)

R 408.802, R 408.803, R 408.814, R 408.829, R 408.831, R 408.832, R 408.839, R 408.839a, R 408.843, R 408.844, R 408.848, R 408.872, R 408.874, and R 408.897 of the Michigan Administrative Codes are amended, R 408.815 is added, and R 408.833, R 408.838 are rescinded as follows:

PART 1. GENERAL PROVISIONSR

R 408.802   Safety requirements for carnival-amusement rides and devices.

Rule 2.  (1)  Electrical wiring, apparatus, and equipment   shall   be manufactured, installed, and maintained as prescribed in the national electrical code (NFPA 70), 2005 2020 edition, published by the national fire protection association.  The provisions of the national electrical code are adopted by reference as electrical standards for the carnival and amusement rides and devices in this state.

(2) These adopted provisions may be purchased for a cost of $72.50  $121.50 at the time  of  adoption  of  these  rules  from  the  National   Fire   Protection Association (NFPA),  Customer  Service  Department, 11Tracy  Drive,  Avon  MA  02322 1 Batterymarch Park, Quincy, MA 02169-7471. Telephone 800/344-3555, (617)/770-3000, website address: www.nfpa.org.  A copy of this code and the general rules of the  board  are  available  for  public inspection at  the  Department  of  Labor  and  Economic  Growth Licensing and Regulatory Affairs,  Bureau  of Construction Codes Commercial Services, 2501 Woodlake Circle, Okemos,  Michigan,  48864,  Phone 611 W. Ottawa, Lansing, MI: (517)/241-9233 9313

(3) The department adopts by reference the ASTM Standards on Amusement Rides and Devices, sponsored by ASTM F-24 Committee, current edition 7th Edition, 2004, as it relates to a carnival or amusement ride as defined by the Carnival-Amusement Safety Act of 1966, 255 PA 1966, MCL 408.651 to 408.670. The standard edition is subject to change as ASTM F-24 Committee publishes new editions.  The adopted standards may be purchased for a cost of $87.00 $338.00 at the time of adoption of these rules from The American Society for Testing and Materials (ASTM), 100 Barr Harbor Drive, P.O.  Box   C 700, West Conshohocken PA  19428-2959, Customer Service, Telephone: (610)-832-9500, website address: www.astm.org. https://www.astm.org/astm-bos-15.07.html . A copy of this code is available for public inspection at Licensing and Regulatory Affairs the  Department  of  Labor  and  Economic  Growth,  Bureau  of Commercial Services Construction Codes, 2501 Woodlake Circle, Okemos,  Michigan,  48864611 W Ottawa, Lansing, MI,  Phone: (517)/241-9233 9313.

(4) The department adopts by reference the standards contained in the American National Standards Institute Safety (ANSI) requirements for aerial passenger tramways, ANSI B77.1 2017, for aerial passenger tramway amusement rides in this state. This adopted standard may be purchased at a cost of $ 200.00 as of the time of adoption of these rules from the American National Standards Institute (ANSI), 25 West   43rd   Street, New York, New York 10036, Phone: (212) 642-4900, Fax: (212) 398-0023 https://webstore.ansi.org/Standards/ANSI/ANSIB772017.

R 408.803    Definitions.

Rule 3. As used in these rules:

(a) "Aerial passenger tramway" means any of the following   devices    used to transport passengers:

(i) "Single and double reversible aerial tramways" which means devices in which passengers are carried in 1 or more enclosed cars that reciprocate between terminals.

(ii) "Chair lift, gondola lift, and monorail" means a device in which passengers are carried on chairs, cars, or in gondola cabs attached to and suspended from a moving wire rope or attached to a moving wire rope or chain and supported on a standing wire rope, rail, or other structure.

(b) “Amusement ride or amusement device” means a device or combination of devices or elements that carry, convey, or direct a person or persons over or through a fixed or restricted course or within a defined area, for the primary purpose of amusement or entertainment. This term shall not include any of the following:

(i) Aerial adventure courses.

(ii) Aquatic play equipment.

(iii) Buildings or concourses used in laser games or paint ball.

(iv) Bungee jumping.

(v) Devices operated on a river, lake, or any other natural body of water.

(vi) Funhouses, haunted houses, and similar walk-through devices that are erected temporarily on a seasonal basis and do not have mechanical components designed to convey patrons.

(vii) Hayrides.

(viii) High speed go-karts that are capable of attaining speeds of 25 miles per hour or more.

(ix) Human-powered devices.

(x) Ice skating rinks.

(xi) Inflatable amusement devices.

(xii) Luge and bobsled tracks.

(xiii) Mechanical bulls.

(xiv) Parasailing.

(xv) Playground equipment, including but not limited to, soft-contained play equipment, swings, seesaws, slides, stationary spring-mounted animal features, jungle gyms, rider-propelled merry-go-rounds, and trampolines.

(xvi) Rock walls.

(xvii) Roller skating rinks.

(xviii) Simulators that can be safely exited mid-ride.

(xix) Skateboard ramps or courses.

(xx) Skydiving simulators.

(xxi) Stationary wave systems.

(xxii) Trains or similar transit devices that can be or previously have been approved for use on the public transit system.

(xxiii) Trampoline courts.

(xxiv) Tubing and sledding hills.

(xxv) Wave pools.

(xxvi) This state reserves the right to include or exclude further devices as presented.

(bc) "Amusement park" means a tract or area used principally as a permanent location for carnival-amusement rides.

(d) “Engineer’s attestation” means a letter signed by a licensed engineer that states the device complies with the applicable act and rules as well as the standards and codes that are adopted by reference in Rule 2, R408.802.

(e) “Gravity rides” means a ride installed on a manmade inclined surface, which  entirely depends on gravity for its operation to convey a passenger from the top of the incline to the bottom in a singular lane, and conveys the passenger in or on a carrier, tube, bag, bathing suit or clothes.

(cf) "Major breakdown" means a stoppage of operation from whatever cause resulting in damage, failure, or breakage of a structural or stress-bearing part of a ride.

R 408.814    Definitions; W.waterslide.

Rule 14. "Water slide Waterslide" means an inclined, nonmechanical   amusement   ride which has all of the following characteristics:

(a) Consists of 1 or more courses of varying slopes and directions.

(b) Relies on gravity and not a mechanical system to propel its passengers.

(c) Uses water to lubricate the course and to regulate passenger velocity. "Water slide Waterslide" does not include slides which are not separate amusement rides but which are customarily recreational equipment as evidenced by a vertical drop of 10 15 feet or less and a total linear slide length of 50 feet or less.

(d) Includes a landing area or runout designed to bring riders to a complete stop and allow them to exit the ride in a safe manner.

R 408.815  Routing  sheets.

Rule 15. Routing  sheets are required to be submitted by mobile operators at the time of application/renewal annually in a paper or digital format.

- Routing sheets must  include all of the following information:

- Specific address where the rides will be set up.

- Setup dates of the rides.

- Operation dates of the rides.

- Number of amusement devices on site if available.

- Specific amusement devices on site, if available.

- Added locations must be updated with the department as soon as      information is available.

R408.816 Operational Training  and Emergency Response Training Forms

Rule 16. Forms to track training for staff operating devices.

- Must be made available to the Department upon request.

- Digital format is acceptable and must contain all information found in paper forms.

- Digital format must be made available upon request.

- Forms must contain the name of company, name of device, state issued device number, staff member(s) trained; date training was conducted; name of trainer; training curriculum; signature of staff trained and the trainer.

PART 2. DESIGN, CONSTRUCTION, AND OPERATION

R 408.829    Seating and carrying devices.

Rule 29. (1) Tubs, cars, chairs, seats,  gondolas,   and   other   carriers used on rides a ride shall be designed and constructed as  strong  as  practical. Their A ride’s interior and exterior parts with  which  passengers   might   come   in contact shall be smooth; rounded; free from sharp,   rough,   or   splintered edges or corners; and have no protruding screws or  projections  which  might cause injury. Parts upon or against which passengers might   be   thrown   by action of the ride shall be adequately padded to prevent  or   minimize   the possibility of injury. The upholstery shall be  kept  in   repair,   and   no loose or flapping portions of upholstery or decoration shall be permitted.

(2) Propellers or other moving parts or decorations   attached   to   tubs, cars, chairs, seats,  gondolas,  and  other  carriers   shall   be   securely fastened to such this equipment and shall be keyed or otherwise  secured  so  that they cannot come off during operation of the  ride.   Vanes, canopies, or other attachments which might become disengaged shall   be   secured   with safety straps to  prevent  their  flying  away  in  case   of   breakage   or dislocation.

(3) Based upon the number of carriers, if a device is shown to have a deficiency or require repairs in the ride’s tubs, cars, chairs, seats, gondolas and other carriers used, the ride shall be deemed to be noncompliant as a whole and shall not operate until returned to compliance. For devices with 10 or fewer carriers, the ride as a whole shall be deemed noncompliant and not operate if 50% or more of the carriers are found deficient.  For devices with more than 10 carriers, the device as a whole shall be deemed noncompliant and not operate if 25% or more of the carriers are found deficient.  This subrule does not apply to go-karts, bumper cars/boats or individual devices that can be removed/replaced as needed such as train cars or log/boat rides.

R 408.831    Safety retainers.

Rule 31. Tubs, cars, chairs, seats, gondolas, and other carriers used on  a ride that depends upon a single means of  attachment or support shall be equipped with a safety retainer to prevent the carrier, if it becomes disengaged from its support or attachment, from being  catapulted  from the ride and to prevent any action  of  the  carrier which might throw the occupants from the carrier while the ride is in motion.   This   rule   only applies to rides or situations determined to be hazardous after inspection by an authorized representative of   the   department   and   review   of   the conditions.

R 408.832    Permit-inspection decal.

Rule 32. (1)  After a ride has been inspected and authorized for operation, the department shall issue a permit-inspection decal which shall be affixed to a basic part of the ride structure in such a manner as to be readily accessible to the authorized inspector. The decal shall set forth the   identification number issued by the department for the individual ride and other information deemed necessary by the department.

(2) If the ride is transferred to another owner/operator, sold, rebuilt, or undergoes major alterations, it shall be reinspected. Upon completion of the inspection, a new permit-inspection decal shall be issued.

(3) If a ride is transferred to another owner/operator or is   sold,   the permit-inspection decal shall be obliterated by  the  owner/operator   before the transfer or sale.

(4) (3) If a decal is mutilated so  that  it  is  no   longer   legible,   the operator shall notify the department and a new decal shall be issued.

R 408.833   Aerial passenger tramways; adoption by reference. Rescinded.

Rule 33.  The  standards  contained  in  the  American  national  standards institute safety requirements for aerial passenger tramways, ANSI B77.1-2006 , are adopted by reference for aerial passenger tramway amusement rides in this state. This incorporated standard may be purchased from the American National Standards  Institute ,  25  West   43rd   Street,   New   York,   NY    10036, Phone:212/642-4900, Fax: 212/398-0023, at a cost of $125.00 as of the time of adoption of these rules.  A copy of this  code,  published  by  the  American National  Standards  Institute,  is  available  for  public   inspection   at www.ansi.org and the Department of  Labor  and  Economic  Growth,  Bureau  of Commercial Services, Carnival-Amusement Safety Board, 2501  Woodlake  Circle, Okemos, Michigan, 48864, Phone: 517/241-9233.

R 408.838    Leveling and alignment. Rescinded.

Rule 38. Corner posts, central columns, or  support   structures    of    a ride designed to operate on a  perpendicular  axis  shall  be    plumb    and secured  so that the path of the sweeps  or  platforms  shall  be  level  and operate on a true horizontal plane at right angles to the   axis    of    the pivot.  A  ride  with carriers  designed to  operate  on  a  horizontal  axis shall be leveled  so  that the carriers orbit in a true perpendicular  plane.The base of a ride that has a combination of orbiting planes or a  ride  that has  carriers  that  operate normally in a plane other than  true  horizontal or vertical shall  be  leveled and plumbed and secured so that they will  not tip or shift and will be stable under the most adverse operating  conditions, except for a  ride  designed   to  permit  flotation    characteristics    or flexibility  or  designed  to  operate properly whether the base is plumb  or level or not.

R 408.839    Gravity rides.

Rule 39.  A ride which is installed on an inclined surface, which depends  on gravity for its operation to convey a passenger from the top of  the  incline to the bottom, and which conveys a passenger in or on a carrier,  tube,  bag, bathing suit, or clothes shall, in addition to other applicable rules, be  in compliance with all of the following provisions: Gravity rides, in addition to other applicable rules, shall comply with all of the following provisions:

(a) The frequency of departure of carriers from the loading area shall be controlled by a ride operator.  The minimum distance or time between departures shall be determined by the director.manufacturer/designer of the device.

(b) The ride shall have an operator located at the loading area and an attendant at the unloading area.

(c) If the entire ride is not visible to the operator, then additional persons shall be located at other stations along the ride to assure complete surveillance of the entire ride. Two-way communication shall be provided between the operator and other attendants of the ride.

R 408.839a   Water slides Waterslides.

Rule 39a. A water slide waterslide that conveys passengers shall, in addition   to other applicable rules, be in  compliance  comply with  all  of  the following requirements:

(a) Each water slide waterslide shall must be designed and constructed to have    a minimum safety factor of 2.

(b) Each water slide  waterslide shall  must be  designed  and  constructed    to    retain the passengers within the water slide waterslide during the ride.

(c) At each loading area, a hard surface which is other  than  earth  and which is reasonably level shall must be provided. The surface shall must be   large enough  to accommodate the intended quantity of passengers.

(d) If the elevation of a loading surface of a water slide waterslide is more    than 12 inches above the elevation of an adjacent  area,  then  guardrails   shall must be installed on the exposed sides of the loading area.

(e) A water slide waterslide may terminate in a swimming pool or  in   a    body    of water such as a lake, river, stream, or artificial  lake  or  reservoir.  The design of the slide waterslide and its termination point shall must meet the requirements  of the  act and these rules.

(f) If the landing area or runout  of  a  waterslide  is  in    a    public swimming pool, the  landing  area  or  runout  of  the  waterslide  shall  be specified by  the designer in  accordance with  R  408.821  of  these  rules. Swimming in this area, other than to exit the  flume  or  landing  or  runout area, is prohibited.

(g) If the landing area or runout of a waterslide is in an  enclosed   tank or in a pool used only to exit the waterslide ride, the  landing   area    or runout shall be specified by the designer in accordance  with  R  408.821  of these rules. This landing area or runout is not a public swimming pool.

(h) The quality of the  water  in  a  swimming  pool   termination    point is governed by rules for public  swimming  pools  promulgated   pursuant   to the provisions of 1978 PA 368,  MCL  333.12522,  and  administered   by   the department of environmental quality. The quality of water in  a   termination point  other than a swimming pool is governed by rules for   bathing    beach water  quality promulgated pursuant to the provisions of 1978 PA   368,   MCL 333.12544,  and administered by the department of environmental quality.

(i h) A water slide shall must have at  least  1  attendant   located    at    the loading area at all times when the ride is being operated for the use of  the public.

A water  slide  attendant  shall  be  trained,  prepared,  and   capable   of controlling the frequency of departure of passengers from the  loading  area. The design and operation of a loading area common to more than 1 course shall must permit an attendant to  remain  knowledgeable  of  the  disposition  of  each passenger. One attendant shall be is sufficient to dispatch riders on more  than 1 course from a common loading area, if a mechanical  system  or  operational controls are used to regulate rider dispatch on each affected course.

(j i) There shall must be at least 1 attendant in the  unloading  area   at    all times when the ride is being operated for the use of the public. An attendant in an unloading area shall must be trained in  cardiopulmonary  resuscitation  and shall must be trained, prepared, and capable of assisting any  passenger   exiting the  water slide waterslide. One  attendant  shall  be  is sufficient  to    operate    the termination  point common to more than 1 water slide, if  the  owner-operator owner/operator the water  slide shall provide provides sufficient attendants in the unloading area to ensure that each attendant can scan the entire area for which he or she is responsible  every 10 seconds and respond within the area within 20 seconds.

(k j) If the entire water slide is  not  visible  to  the  attendant  at  the loading area or if the attendant is controlling entry  to  multiple  courses, then  some  form  of  2-way  communication  shall  must be  provided  between  the attendants  of  the ride to ensure that passenger loading is stopped if there are problems.

R 408.843    Personal injuries and deaths. Rescinded.

Rule 43. (1) Provision shall be made to render first aid    when    persons are injured on or around any ride.

(2) An owner/operator  shall  report,  in  writing  to   the    department, an accident  resulting  in  injury  to  any  person  within  24  hours  after occurrence.

(3) In addition to the  requirements  of  subrule  (2)  of    this    rule, the owner/operator shall report to the department  by   telephone any accident in  which  a  fatality  occurs   or   a   person    suffers    a fracture, concussion,  laceration,  or  other  traumatic   injury   requiring immediate surgical or medical care resulting from  structural  or  mechanical failure of a ride, or in which it might appear that the construction, design, or  function of the ride is directly involved. If an accident  specified   in this  subrule occurs on a non-workday of the department,  the  owner/operator shall report the incident to the department by  9:00  a.m.of the next business  day.   The  department,  after  consultation  with  the owner/operator, may require that the scene of an accident be secured and  not disturbed to any greater extent than necessary for removal of the deceased or injured persons. If a ride is removed from service due to  an  accident,  the department shall order an immediate investigation of the secured site and the ride shall be released for repair and operation only after the  investigation is completed.

R 408.844    Mechanical failure reports.

Rule 44. The owner/operator of a  carnival-amusement  ride  shall  report a major breakdown to the department within 24 hours  after  occurrence   of the incident  by  telephone  or  other  media  means of  immediate  communication. The owner/operator shall confirm this report  using  the  form  provided   by the department. This report shall be forwarded to the department   within   7 days after the occurrence of the reportable incident.  Upon   being   advised of  a major breakdown, the department may  order  the  ride  to  be  withheld from  operation  and  the  department   shall    conduct    an     immediate investigation.  The ride shall be released  for  repair  and  operation  only after completion of  the department investigation. the department may require that the scene of an accident, major breakdown, or both be secured and not disturbed to any greater extent than necessary for removal of the deceased or injured persons. The department may order an immediate inspection of the secured site and the ride shall be released for repair and operation only after the inspection is completed.

R 408.848    Control and operation.

Rule 48. (1) A ride shall be operated by a person who is trained to operate the ride.  The operator of a ride designed for the exclusive use of children and the operator of other rides for which the operator does not have mechanical or electrical controls shall be not less than 16 years of age.  For all other rides, an operator shall be not less than 18 16 years of age.

(2) An operator shall have knowledge of the use and function of normal operating controls, signal systems, and safety devices applicable to the ride and of the proper use, function, capacity, and speed of the particular ride at all times that it is being operated. When the ride is shut down, provision shall be made to prevent the ride from being operated by the public. A person other than a trained operator shall not be permitted to handle the controls of a ride during normal operation, except where the ride is designed to be controlled by the passenger.

PART 3. PROCEDURES

R 408.872    Emergency applications for permits to operate.

Rule 72. When an owner/operator has not previously intended to operate a ride in this state and has not made application for a permit   to   operate, the owner/operator, upon confirming a Michigan booking, shall   notify   the department and apply for a permit to operate. The notice   shall   be   given not less than 30 days before the book  date.  When   an   emergency   booking makes the 30-day notice impossible, the owner/operator   shall   notify   the department of the booking by  telephone  or  other   means   of   immediate communication and shall confirm this notice in writing.  The  director  shall schedule and arrange for inspection of the rides and  the   issuance   of   a permit to operate in conjunction with the owner/operator to best serve the needs of both parties as will best serve the needs of  the   owner/operator   and the orderly administration of the act and these rules. If devices require inspection before operation, then the owner/operator shall make every effort to arrive at a location within this state and set up the devices to be inspected.

R 408.874    Daily inspection.

Rule 74. A ride shall be inspected and tested on each day it is intended to be used. This inspection shall be  made   by   a   person   commissioned   to perform  such  inspections  by  the  director.  Physical or digital Rresults   of   these    daily inspection  and  performance  tests  shall  be performed and recorded   in   the   a manner prescribed by the manufacturer or ASTM F-24 if manufacturer standards are unavailable, and  on  forms  provided,  by  the  department   and   shall   be certified by the person commissioned to perform  these   inspections.   These inspection reports shall be retained at the operation site  until  the   next inspection is conducted by  the  director.  An   owner/operator   shall   not knowingly use, or permit  to  be  used,  a  ride  which   is   not   properly assembled or which is defective or unsafe in any of its  parts,  controls, or safety equipment.

PART 4. PARTICIPATORY RIDES--GO-KARTS

R 408.897   Remote idle system.

Rule 97. (1) Effective May 1, 2000, each go-kart shall be equipped with a receiver for a remote idle system. A go-kart ride attendant shall control the remote idle system.

(2) If the remote idle system becomes inoperable and requires repair, then the track owner-operator shall make a written request to operate the go-kart rides until the repair is completed. The duration of the repair period shall not be more than 30 days.